# New Mexico Consumptive Use Analysis

# Consumptive Use for the Gila/San Francisco Rivers

Updated October 2022

Contents

Introduction and Accompanying Spreadsheets	2

Introduction	2

Weather	5

CropStats	5

Diversion Records	6

FORTRAN Program (XirrigCU)	7

Crop Coefficient File	8

Weather Data File	9

Crop Data File	10

Modified Blaney Criddle (MBC) Control	12

Original Blaney Criddle (OBC) Control	13

Running XirrigCU	14

Analyzing the Output Files	15

OBC Cropsum	15

MBC Cropsum	16

OBC Regionsum	18

MBC regionsum	18

Final QA/QC	18

Gila Rpt20xx	19

Reporting letter	25

Miscellaneous	26

Troubleshooting XirrigCU	26

Irrigated acreage locale and their associated weather stations	27

# Introduction and Accompanying Spreadsheets

## Introduction

[Insert some block of text about the decree]

##### Irrigation methods

- Original Blaney Criddle Formula (OBC):

The OBC method (Blaney and Criddle, 1950, 1962) for determining consumptive use was born out of studies conducted in New Mexico during 1939 and 1940 for the Pecos River Joint Investigation initiated by the National Resources Planning Board.

The OBC method uses mean monthly air temperatures (t) expressed in degrees Fahrenheit, monthly percentage of annual daylight hours (P) based on latitude of the area under study, seasonal consumptive use coefficients (K), and length of growing season.  These are used to estimate the total consumptive use (ETc), or evapotranspiration (ET), of water during the growing season for a crop.  The consumptive use in inches for each month is expressed as:

ETc=KF

Where: ETc = estimated ET (in inches)

K = empirical monthly crop coefficient

F = sum of monthly CU factors f

f = monthly CU factor (tP/100)

t = mean monthly temperature

P = percent of daylight hours

Adding the consumptive use computed for each month yields the total consumptive use for a specific crop during the growing season.

The distinctive feature of this method is that the consumptive use coefficient (K) remains constant throughout the frost-free period. A different consumptive use coefficient is used for that part of a crop’s growing season that occurs before the last spring frost (T<32F) or past the first fall frost (T>32F).  The consumptive use coefficient during the frost period is lower than the consumptive use coefficient during the frost-free period of the growing season.  For crops that have a growing season that begins before or extends beyond a frost date—in a month in which a frost occurs—the days inside and outside the frost-free period must be counted separately so that the appropriate consumptive use coefficients can be applied. In a month during which the growing season begins or ends, the consumptive use coefficient is multiplied by the ratio of the number of days in the month the crop is “growing” to the total number of days in that month.

- Modified Blaney Criddle Formula (MBC):

The U.S. Department of Agriculture (USDA) Soil Conservation Service (SCS) introduced the MBC method in 1967. In 1970 it was revised and published (USDA, 1970).  The MBC method uses mean monthly air temperatures (t) expressed in degrees Fahrenheit, monthly percentage of annual daylight hours (p) based on the latitude of the area under study, monthly consumptive use coefficients (k), and length of growing season to estimate the total monthly consumptive use (um) of water for a crop that is well watered and free of disease. The consumptive use in inches for each month is expressed as:

ETc=KF

Where: ETc = estimated ET (in inches)

K = Kt * Kc

Kt = 0.0173t–0.314

Kc = Crop coefficient (growing stage)

F = sum of monthly CU factors f

f = monthly CU factor (tP/100)

t = mean monthly temperature

P = percent of daylight hours

The procedure used to calculate the final monthly consumptive use coefficient (k) distinguishes the MBC method from the OBC method, which uses a seasonal consumptive use coefficient.

- The climatic coefficient (kt), expressed as a function of the mean monthly temperature, is computed (kt= 0.0173t–0.314)

- The value of the crop growth stage coefficient (kc) is obtained from a curve plotted on a graph. Because the growth characteristics of each crop are different, a separate curve is generally required for each crop. Curves for a limited number of crops were published in SCS Technical Release 21 (USDA, 1970).

- Multiplying kt by kc yields the final consumptive use coefficient (k) for a specific crop and month.

- In a month in which the growing season begins or ends, the consumptive use coefficient is multiplied by the ratio of the number of days in the month the crop is “growing” to the total number of days in that month.

- Adding the consumptive use computed for each month yields the total consumptive use for a specific crop during the growing season.

##### Effective precipitation calculations:

- USBR Effective Precipitation calculation:

The amount of rainfall available to crops is influenced by the following factors:

- Duration and intensity of rainfall

- Antecedent moisture condition of the soil

- Infiltration capacity of the soil

- Presence of surface seals and crusts

- Slope of fields

- Root development of the crop

- Interception by the plant canopy

The OBC method did not have a procedure for estimating effective rainfall, and in 1962, Blaney and Criddle adopted the USBR method.  The USBR method expresses effective rainfall (Re) as a percentage of the total monthly rainfall.  For each 1-inch increment in total monthly rainfall (Rt), there is a corresponding decrease in the percentage of effective rainfall.  Note: Effective rainfall (Re) cannot exceed the consumptive use.  Adding the effective rainfall computed for each month yields the total effective rainfall for a specific crop during the growing season.

|  | Re = 0.95 * Rt | Rt < 1.0 inch |  |
| --- | --- | --- | --- |
|  | Re = 0.90 * (Rt – 1.0) + 0.95 | 1.0 < Rt < 2.0 inches |  |
|  | Re = 0.82 * (Rt – 2.0) + 1.85 | 2.0 < Rt < 3.0 inches |  |
|  | Re = 0.65 * (Rt – 3.0) + 2.67 | 3.0 < Rt < 4.0 inches |  |
|  | Re = 0.45 * (Rt – 4.0) + 3.32 | 4.0 < Rt < 5.0 inches |  |
|  | Re = 0.25 * (Rt – 5.0) + 3.77 | 5.0 < Rt < 6.0 inches |  |
|  | Re = 0.05 * (Rt – 6.0) + 4.02 | Rt > 6.0 inches |  |

- SCS Effective Precipitation calculation:

The SCS developed a method for estimating effective rainfall as a function of consumptive use and rainfall.  This method was established as the result of research that analyzed the soil-moisture balance from 50 years of precipitation records at each of 22 Weather Bureau stations in the United States (now part of the National Weather Service).  To calculate effective rainfall (re) in inches, the SCS method uses the following equation:

Re=(0.7091Rt0.82416-0.11556)(100.02426 cu)

Where: Re = Effective Precipitation

Rt = total monthly precipitation

Cu = average monthly consumptive use

F = a function of D

D = Net depth of application (assumed always 3”, so F = 1)

F = 0.531747 + 0.295164(D) – 0.57697(D2) + 0.003804(D3)

In New Mexico, the default depth of irrigation is three inches.  Note: The effective rainfall (re) cannot exceed the monthly consumptive use.  The monthly CIR for each crop in the cropping pattern is computed by subtracting the effective rainfall (Re) from the consumptive use.  Adding the computed effective rainfall for each month yields the total effective rainfall for a specific crop during the growing season.

For the Gila area, the preferred methods are the OBC and USBR Effective Precipitation methods.  The results will be interpreted in greater detail in Analyzing the Outputs.

## Weather

Inputs for both versions of Blaney Criddle and effective precipitation require the same weather data - mean monthly temperatures, total monthly rainfall, and the dates of the first and last frosts in each year.  These data are obtained every year from federally recognized meteorological entities like NOAA or NWS.  Citizen science weather stations like Weatherunderground or CoCoRahs should be avoided wherever possible.

Data sources:

- Climate pdfs: https://www.ncdc.noaa.gov/IPS/cd/cd.html

This NOAA site contains monthly and annual pdfs for all 50 states and was the basis of the CU program.  It contains all three parameters needed and will highlight error flags.  Most of the sites still exist and contain a good record

- Raw data behind the climate pdfs: https://www.ncdc.noaa.gov/cdo-web/search

Raw data can be downloaded, and other stations examined if needed.  Open the CDO web search, in ‘Select Weather Observation Type/Dataset, select ‘daily summaries’.  Make sure the date selected extends to the current date. There are multiple ways to select the required information here, but for this example use ‘Search for’ select ‘Hydrologic Cataloging units’.  Next, enter ‘15040001, 15040002, 15040004, 15040006’ as the search term.  These are 6 digit HUC codes, and will return the station list for the Upper Gila, Middle Gila, San Francisco, and San Simon areas.  Navigate through the station lists to find the parameters needed.

- Mesowest: https://mesowest.utah.edu/cgi-bin/droman/mesomap.cgi?state=NM&rawsflag=3

This is a free site but making an account will help greatly.  It aggregates weather data from numerous NWS and RAWS weather stations.  Find New Mexico, and have it search for NWS and RAWS stations on the map.  You can download a full year of data for each site and run the monthly totals.  Arizona data may also be obtained here for the border areas.

- WRCC DRI site: https://wrcc.dri.edu/wraws/nmF.html

This is another aggregator of RAWS data, but instead of outputting the data as a data file, it will output an html that will have some total statistics pre-generated.

- ClimateEngine - https://app.climateengine.com/climateEngine

This site aggregates a lot of remote sensing and weather data to provide multiple kinds of analysis types.  For this process, use the PRISM monthly datasets for a point at the location of interest.  Switch to minimum daily PRISM temperature to find the frost dates.

For the Gila area, the sheet will contain 9 areas that require annual data updates using the sources listed above. Update the workbook accordingly.

## CropStats

[Insert some block of text about how to do the cropstats]

First, ensure that the setup of the CropStats file is in a table format so you can filter multiple columns at once.  Highlight all the cells in the table and click insert -> insert table.  Then go to Table Design and click the box for ‘Total Row’.  This will add a new row at the bottom.  Navigate to the bottom of the ‘computed acreage’ column and click the dropdown.  This will allow you to pick ‘sum’ as the calculation type.  Adding data together manually in the margins is inefficient.

## Diversion Records

[Insert some block of text about the gages? Or reference another document about that?

Of the 23 gages…

Open an older ‘Ditch Diversions 20xx’ spreadsheet

# FORTRAN Program (XirrigCU)

The Fortran program XirrigCU is the current method for assessing the State of New Mexico’s consumptive irrigation use.  It is written to process irrigation consumptive use along the Gila/San Francisco using the Original Blaney-Criddle (OBC) and Modified Blaney Criddle (MBC) methods.  In addition, it calculates effective precipitation using the USBR and SCS methods.  For this process, there are 5 input text files required written in legacy formats that require detailed instructions to understand them.  The following will detail the construction and use of these input files, and help the user understand the outputs and troubleshoot any errors that may arise in processing.

MS Word comment bubbles were used to explain the differences between the values while preserving the formatting of the input files. Those pages are best printed separately from the rest of the document.

- Crop Coefficient File (CC)

This file holds the crop coefficient values for 30 crops, as well as start-up and cutoff dates for crops and locales.  The editable portion of the file is the cutoff dates in 7 of the crop types and is copy and pasted from year to year.  Only a select few of these cutoffs are used, but all require updating.  No external data entry is required unless a hard cutoff was imposed during irrigation season.

- Weather Data File

This file contains the mean monthly temperatures and precipitation for the areas listed as well as the first and last frost dates.  Each station requires an annual update from the Weather Data spreadsheets.

- Crop Data File (CDF)

Crop survey data that has been processed in the Crop Stats spreadsheet will end up here.  Annual accounting of irrigated acreages, crop types, and irrigation type will all be entered according to the layout described.  The Gila area file is inclusive of both Gila and San Francisco basins.  The San Juan files are separated into San Juan, Animas, and La Plata rivers.

- Modified Blaney-Criddle Control

The MBC control file will name the input files (.dat) described above and name the output files (.out) which will contain the final MBC CU numbers.  It will select the MBC method for CU, and the SCS method for effective precipitation.  The only entry required is changing the years.

- Original Blaney-Criddle Control

The OBC control file will name the input files (.dat) described above and name the output files (.out) which will contain the final OBC CU numbers.  It will select the OBC method for CU, and the USBR method for effective precipitation.  The only entry required is changing the years.  This is the preferred output for use in the Gila-San Francisco basin.

## Crop Coefficient File

The following is an example of the Crop Coefficient File, GilaAreaCC’68-’20.DAT.

30       1      30

1    Alfalfa

PR       50.   28.  0.85  0.50  365.

1.   15.   32.   46.   60.   74.   91.  105.  121.  135.  152.  166.  182.

196.  213.  227.  244.  258.  274.  288.  305.  319.  335.  349.  366.

0.60  0.63  0.68  0.73  0.79  0.85  0.92  0.99 1.045  1.09  1.12 1.135  1.13

1.115  1.09 1.065  1.03  0.99  0.95 0.905  0.85  0.79  0.72  0.64  0.60

0

1    2020       6      10       9      30

2    2020       6      10       9      30

3    2020       6      10       9      30

4    2020       6      10       9      30

5    2020       5      20      10       1

6    2020       5      20      10       1

7    2020       5      20      10       1

8    2020       5      15       9      15

9    2020       5      15       9      15

The CC file is broken into 30 crop types.  Each crop type has the crop coefficients listed for both OBC and MBC (see comments for this delineation).  The lines with the years are showing potential start up and cut off dates that can be applied to various crops.  Not all crops will have an associated date table.  This file will require an annual update by the user before running.  A current year line should be added to every section containing dates.  No other updates are required.

The crop coefficient values in this file have been copied into an accompanying spreadsheet for examination and comparison.  While transferring crop data from the crop stats workbook to the CDF file, it may be useful to compare the values of one crop type to another.

## Weather Data File

Below is an example of the ‘GilaAreaWeather'68-'20.DAT’.

1      LUNA (USES LUNA RANGER STATION)

1       1

33.49

[…]

2020  1.20  1.82  1.62  0.35  0.12  0.05  0.09  1.61  0.05  0.52  1.16  0.42

31.35   31.90

[…]

2020  31.3  34.9  40.0  47.7  57.9  64.8  67.1  66.6  59.6  49.6  39.7  28.1

[…]

2020       5      24       6      15       9      12      10      19

Copy and paste in the current years mean temperature, monthly precipitation, and frost dates in three different places.  If copying from excel, delete out and manually re-enter any spaces between values.  Preserve the decimal place holders, and either round to or add zeros to do this.

## Crop Data File

Below is an example of the ‘GilaAreaCDF'68-'20.DAT’. Updating the CDF file requires a completion of the CropStats spreadsheet where the associated cropping acreages can be found.  There are three main areas that require updating on an annual basis: the total acreage line, the cropping pattern line, and the irrigation type line.

1      LUNA LOCALE

2      APACHE-ARAGON LOCALE

3      RESERVE LOCALE

4      GLENWOOD LOCALE

5      UPPER GILA LOCALE

6      CLIFF-GILA LOCALE

7      REDROCK LOCALE

8      VIRDEN VALLEY LOCALE

9      SAN SIMON LOCALE

1      LUNA

33.49

1       7       1       1

5       3      28      29       6       1      25

0.

[…]

2019   108.1     0.0     0.0     0.0     0.0     0.0

0.0     0.0     0.0    98.9     0.0     0.0     1.1

[…]

2019    99.1    10.0     0.9     2.0     0.0     0.0     0.0

0

Data Entry: There are three areas that require an update in each locale (See comments).  First, the Crop surveys may vary slightly between the surveyors, for example, one surveyor might identify a field as pasture (native) while another might use the pasture (unimproved) designation.  This may create a problem on the fortran data entry, if the ‘new’ crop type was never mapped.  There are two methods for dealing with this.  First, examine the crop coefficients found in the accompanying spreadsheet mentioned in the CC file above.  If there is already a mapped crop type with an identical crop coefficient (pasture native and pasture unimproved are nearly identical), then the ‘new’ crop type can be subbed in for the ‘old’ and there is no difference in the calculations.  If the crop coefficients are different, it will need to be mapped into the sheet.

Adding new crops

Using the example above, first change the ‘7’ to an ‘8’ to signify that there are now 8 crop types present in the data (see comment 4 on this page).  Next, while observing the existing space formatting, append the crop type map with the new CC value at the end (comment 5).  If the new value is an 11th new crop, start a new line below it.  As a new crop is added, a 0.0 percent value should be placed in the data for each year starting from the beginning of the file.  This ensures that the integrity of the past data is maintained, and that those years can be re-run if necessary.

| Lat | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 7.5 | 7.1 | 8.4 | 8.6 | 9.3 | 9.2 | 9.4 | 9.1 | 8.3 | 8.1 | 7.4 | 7.4 |
| 26 | 7.5 | 7.1 | 8.4 | 8.6 | 9.4 | 9.3 | 9.5 | 9.1 | 8.3 | 8.1 | 7.4 | 7.4 |
| 27 | 7.4 | 7.1 | 8.4 | 8.7 | 9.4 | 9.3 | 9.5 | 9.1 | 8.3 | 8 | 7.3 | 7.3 |
| 28 | 7.4 | 7.1 | 8.4 | 8.7 | 9.5 | 9.4 | 9.6 | 9.2 | 8.3 | 8 | 7.3 | 7.3 |
| 29 | 7.4 | 7.1 | 8.4 | 8.7 | 9.5 | 9.4 | 9.6 | 9.2 | 8.3 | 8 | 7.2 | 7.2 |
| 30 | 7.3 | 7 | 8.4 | 8.7 | 9.5 | 9.5 | 9.7 | 9.2 | 8.3 | 8 | 7.2 | 7.2 |
| 31 | 7.2 | 7 | 8.4 | 8.7 | 9.6 | 9.6 | 9.7 | 9.2 | 8.3 | 8 | 7.2 | 7.1 |
| 32 | 7.2 | 7 | 8.4 | 8.8 | 9.6 | 9.6 | 9.8 | 9.3 | 8.3 | 8 | 7.1 | 7.1 |
| 33 | 7.2 | 6.9 | 8.4 | 8.8 | 9.7 | 9.6 | 9.8 | 9.3 | 8.4 | 7.9 | 7.1 | 7 |
| 34 | 7.1 | 6.9 | 8.4 | 8.8 | 9.7 | 9.7 | 9.9 | 9.3 | 8.4 | 7.9 | 7 | 6.9 |
| 35 | 7 | 6.9 | 8.4 | 8.8 | 9.8 | 9.8 | 9.9 | 9.4 | 8.4 | 7.9 | 7 | 6.9 |
| 36 | 7 | 6.8 | 8.4 | 8.8 | 9.8 | 9.8 | 10 | 9.4 | 8.4 | 7.9 | 6.9 | 6.8 |
| 37 | 6.9 | 6.8 | 8.3 | 8.9 | 9.8 | 9.9 | 10.1 | 9.4 | 8.4 | 7.8 | 6.9 | 6.7 |
| 38 | 6.9 | 6.8 | 8.3 | 8.9 | 9.9 | 10 | 10.1 | 9.5 | 8.4 | 7.8 | 6.8 | 6.7 |
| 39 | 6.8 | 6.8 | 8.3 | 8.9 | 10 | 10 | 10.2 | 9.5 | 8.4 | 7.8 | 6.8 | 6.6 |

This is the 'percent daylight hours' table and can be found at the bottom of the CDF file. This shows the percentage of annual daylight hours that occur in each month. Each row will add up to roughly 100 percent. For example, at 33 degrees latitude, the month of June will have 9.6% of the annual daylight hours per year.  The original table is from the Tech Report 32 (table B4). This is used in the main calculation (See cropsum outputs below).

Note that the fortran file itself does not contain headers, these were added here for demonstration.

## Modified Blaney Criddle (MBC) Control

Gila-San Francisco 1968-2020 CU xirrigcu program MBC SCS rain

GilaAreaCDF'68-'20.DAT

GilaAreaWeather'68-'20.DAT

GilaAreaCC'68-'20.DAT

1       1       0       0      68      45

gila'20mbc-scs_cropcu.out

gila'20mbc-scs_regionsum.out

gila'20mbc-scs_mondep.out

gila'20mbc-scs_alfunc.out

2020    2020       9       9       9    1968    2020

0       1       0       2

In comment 5 of this page, the first two values set the start and stop dates for the program.  Setting the start ahead of the current year will extend the record for processing.  The earliest Gila year is 1968, and the earliest San Juan is 1929.  The program can be run for the entire record in a few seconds more of processing time. This can be valuable for error checking but is not the only step in post processing.

All the years will need to be changed to reflect the new input and output files, as well as the years to be calculated.

## Original Blaney Criddle (OBC) Control

Gila-San Francisco 1968-2020 CU xirrigcu program OBC usbr rain

GilaAreaCDF'68-'20.DAT

GilaAreaWeather'68-'20.DAT

GilaAreaCC'68-'20.DAT

1       1       0       0      68      45

gila'20obc-usbr_cropcu.out

gila'20obc-usbr_regionsum.out

gila'20obc-usbr_mondep.out

gila'20obc-usbr_alfunc.out

2020    2020       9       9       9    1968    2020

1       0       0       1

In comment 5 of this page, the first two values set the start and stop dates for the program.  Setting the start ahead of the current year will extend the record for processing.  The earliest Gila year is 1968, and the earliest San Juan is 1929.  The program can be run for the entire record in a few seconds more of processing time. This can be valuable for error checking but is not the only step in post processing.

All the years will need to be changed to reflect the new input and output files, as well as the years to be calculated.

# Running XirrigCU

After the five input files described above have been updated, it is time to run XirrigCU.  Place each file into the same working directory (folder) along with any copy of the xirrigcu-10-06.exe program.  Open XirrigCU. Upon running, you will be prompted with a command line asking you to:

You will copy and paste the filename to one of the control files and push Enter.  In this example, we have used the MBC control file:

Gila'20mbc-scsControl.DAT

The program will run and then close itself, creating two output files (.out) in the working directory as described by the Control file.  Repeat the process for the OBC control file:

Gila'20obc-usbrControl.DAT

XirrigCU will close again, and you will have two more new files in your working directory.  The process now is complete, and the results are detailed in the following files:

Modified Blaney Criddle

- gila'20mbc-scs_cropcu.out

- gila'20mbc-scs_regionsum.out

Original Blaney Criddle

- gila'20obc-usbr_cropcu.out

- gila'20obc-usbr_regionsum.out

Total run time for XirrigCU is less than a second.

XirrigCU creates a Cropsum and Regionsum file for each CU method, and the sum files are functionally identical to each other, but with different results.  The Cropsum file details the analysis and show the calculations for each crop. The regionsum takes the final answer and applies it to the region.

# Analyzing the Output Files

## OBC Cropsum

This file contains numerous text-based tables that detail the results of the analysis. The formatting can be seen in the bullets below, and are nested for every crop type, year, and locale.  If the user has selected multiple years in the run, the output will contain even more tables but will be organized in similar formatting.  The table numbering starts over with every successive year, and it is not instructive to use table numbers for this example.  Tables 1 through n are crops, and tables n+1 and n+2 denote the last two tables after the crop types in each area.

- Irrigated Area –

  - Year (starting year specified in Control File)

    - Table 1 (First crop listed in CDF)

    - Table 2 (Second crop listed in CDF)

    - […]

    - Table n (Last crop listed in CDF)

    - Table n+1 (Weighted Annual CIR For Cropping Pattern)

    - Table n+2 (Weighted Monthly CIRS in Inches For Cropping Pattern)

  - Year […]

  - Year (last year specified in control file)

    - Table 1 (First crop listed in CDF)

    - Table 2 (Second crop listed in CDF)

    - […]

    - Table n (Last crop listed in CDF)

    - Table n+1 (Weighted Annual CIR For Cropping Area)

    - Table n+2 (Weighted Monthly CIRS in Inches For Cropping Pattern)

The OBC cropsum will detail the results using original Blaney Criddle and USBR Effective Precipitation.

In the example table below, the left side shows the months and number of days within the growing season.  Light and temperature are adjusted numbers, based on the number of days in the month.  This happens in a calculation outside of this table and truncates the values to represent a partial month correctly.  The resulting temp and light fields are presented here and used as inputs toward the f value.  To get f, each month is expressed as (temp * light)/100.  Each f is multiplied by the K value (Kiff for in the growing season, Koff for out of growing season, values are found in the CC.DAT file) and that will yield the ETc for that month.  The total sum of those months equals the consumptive use (ETc in inches) for the year.  (Example Luna, OBC and USBR rain in 2016):

Effective rainfall, Re, is calculated using the USBR method.  The first inch of monthly precipitation is assumed to be 95% effective for irrigation, and each subsequent inch is assumed to be less effective by a downscaling factor (see Introduction for method).  The Re figure is also truncated by the shorter beginning and ending months and may not calculate out exactly as described.  The Re field dictates the results of that analysis.

The final results for the crop are shown in the last three columns, the ETc, Re, and CIR.  ETc calculated by the Blaney Criddle Method is summed for each month, and Re is calculated by the USBR method and summed for each month.  The total Consumptive Irrigation Requirement (CIR) for that crop is then calculated as CIR = ETc – Re.

EXAMPLE TABLE  1. MONTHLY CIR FOR:     Pasture (Improved)

GROWING SEASON: MAY  6 TO OCT 29, LENGTH 177 DAYS

================================================================

MONTH DAYS TEMP  RAIN LIGHT     f KIFF KOFF   ETc    Re   CIR

================================================================

MAY     26 48.2  0.64   8.1  3.92      0.50  1.96  0.51  1.45

JUN     30 60.8  1.22   9.7  5.89 0.80 0.50  3.71  1.15  2.56

JUL     31 63.5  1.54   9.9  6.27 0.80       5.02  1.44  3.58

AUG     31 58.2  5.06   9.3  5.41 0.80       4.33  3.78  0.55

SEP     30 53.4  1.98   8.4  4.49 0.80 0.50  2.92  1.83  1.08

OCT     29 49.9  1.20   7.4  3.69      0.50  1.84  1.06  0.79

TOTAL 177      11.64                       19.78  9.77 10.01 IN

0.83 FT

Table n+1 follows the last crop type listed in the CDF and is the summary for the irrigated area.  It lists each crop type, the acres of each crop, the Crop Distribution Ratio (CDR), and the CIR as calculated for each crop in the preceding tables.  The CDR is a weighting factor used to adjust the final weighted CIR (WCIR) as WCIR = CDR * CIR.  Make sure that the CDR is 1.000, or else the data is incorrect, and there is no warning produced.

TABLE n+1. WEIGHTED ANNUAL CIR FOR CROPPING PATTERN

==================================================================

CROP                        ACRES      CDR      CIR     WCIR

==================================================================

Pasture (Improved)        0.0    0.000    12.27     0.00

Pasture (Unimproved)    149.9    0.944    11.36    10.72

Alfalfa                   0.0    0.000     8.86     0.00

Turfgrass (Warm Season)   8.9    0.056     8.71     0.49

TOTAL    158.8    1.000             11.21 IN

The final table in each irrigated area is the weighted monthly CIR.  This table partitions out the consumptive use over the growing season.

TABLE  n+2. WEIGHTED MONTHLY CIRS IN INCHES FOR CROPPING PATTERN

JAN   FEB   MAR   APR   MAY   JUN   JUL   AUG   SEP   OCT   NOV   DEC   TOTAL

0.00  0.00  0.00  1.10  1.55  2.58  0.88  1.86  0.91  1.73  0.61  0.00  11.21

## MBC Cropsum

This file contains the same makeup of tables seen in the OBC cropsum. The formatting can be seen in the bullets below, and are nested for every crop type, year, and locale. Multiple year runs will contain even more tables but will be organized in similar formatting.  Between irrigated areas, the table numbers are not constant since each area has different numbers of crops within them.  The n+1 and n+2 designations simply denote the last two tables after the crop types in each area.

- Irrigated Area –

  - Year (starting year specified in Control File)

    - Table 1 (Crop 1 listed in CDF)

    - Table 2 (Crop 2 listed in CDF)

    - […]

    - Table n (Last crop listed in CDF)

    - Table n+1 (Weighted Annual CIR For Cropping Pattern)

    - Table n+2 (Weighted Monthly CIRS in Inches For Cropping Pattern)

  - Year […]

  - Year (last year specified in control file)

    - Table 1 (Crop 1 listed in CDF)

    - Table 2 (Crop 2 listed in CDF)

    - […]

    - Table n (Last crop listed in CDF)

    - Table n+1 (Weighted Annual CIR For Cropping Area)

    - Table n+2 (Weighted Monthly CIRS in Inches For Cropping Pattern)

In the example table below, the months to be calculated are presented with the number of days within the growing season on the left.  Light and temperature are adjusted numbers, and the values may vary based on partial months.  This happens in a calculation outside of this table and truncates the values to represent partial months correctly.  The resulting temperature and light fields are presented here and used as inputs toward the f value.  To get f, each month is expressed as (temp * light)/100.  Kt is calculated as defined in the equation from the introduction.  Kc is an empirical crop coefficient value based on the growing stage of the crop.  The final ETc is then completed as ETc = (Kt*Kc) * f.

TABLE  1. MONTHLY CIR FOR:     Pasture (Improved)

GROWING SEASON: APR 10 TO OCT 28, LENGTH 202 DAYS

================================================================

MONTH DAYS TEMP  RAIN LIGHT     F   Kc   Kt   ETc    Re   CIR

================================================================

APR     21 47.1  0.60   6.2  2.90 0.86 0.50  1.25  0.26  0.99

MAY     31 50.3  0.23   9.7  4.88 0.90 0.56  2.44  0.11  2.34

JUN     30 61.2  0.52   9.7  5.93 0.92 0.74  4.06  0.37  3.69

JUL     31 67.1  6.66   9.9  6.63 0.92 0.85  5.19  4.37  0.82

AUG     31 65.5  2.58   9.3  6.09 0.90 0.82  4.51  1.85  2.67

SEP     30 61.2  0.87   8.4  5.14 0.87 0.74  3.33  0.62  2.71

OCT     28 51.0  0.66   7.1  3.64 0.80 0.57  1.65  0.38  1.26

TOTAL 202      12.12                       22.44  7.97 14.47 IN

1.21 FT

Similar to OBC, the resulting data value is CIR = ETc – Re.  This is the total depth of depletion for that crop, in that area, for that year.

## OBC Regionsum

This output contains one table for each irrigated area and the final consumptive use results for each year calculated.  It shows the acreage (total acres from CDF) and the weighted CIR from the cropsum output.  It calculates the crop CU full supply as acres * WCIR.  If any shortages were claimed, it will reduce those values in the Consumptive use field.  Incidental Depletions are calculated and are added to the total irrigation depletion for the irrigated area.

This regionsum table formatting is far more conducive for excel than the cropsum file.  Copy and paste the outputs into a spreadsheet and compare the current year outputs to previous years.

## MBC regionsum

This output contains one table for each irrigated area and the final consumptive use results for each year calculated.  It shows the acreage (from CDF) and the weighted CIR from the cropsum output.  It calculates the crop CU full supply as acres * WCIR.  If any shortages were claimed, it will reduce those values in the Consumptive use field.  Incidental Depletions are calculated (somehow) and are added to the total irrigation depletion for the irrigated area.

# Final QA/QC

Ensure that the CDR values for each crop in cropsum are 1.000.  Use “crtl+f’ in the file to find all the CDR values.  This will be the most efficient check.  Run XirrigCU for multiple years and copy the regionsum tables into an excel sheet.  Pivot tables are useful to examine the values.

## Gila Rpt20xx

This workbook consists of several spreadsheets designed to calculate the final tables and values for reporting.  This is to be completed after all other sheets and XirrigCU are done.  The calculations it preforms are simple, but the sheet cross references itself in very exacting ways that should not be altered.  In this, it accomplishes multiple things: it calculates the consumptive use from various water rights sectors like livestock, domestic, and industrial uses.  It compiles the irrigation consumptive uses calculated by XirrigCU and assesses any shortages that may have occurred during the season based on the diversion records.  Finally, it takes all that information and compiles it into tables ready for reporting based on irrigation area and use type.

All spacings, values, and formatting are critical to maintain.  Only values written in color (i.e., not black) will require annual updates.  There is a practice to turn all those values green in one year, and then red the next, and then back again so the operator can keep their place.  Make sure it is noted in the sheet what color the current year is, and which is the previous year.  Those notes can be deleted when finished.

The following are data sources needed to complete the workbook:

- USDA’s annual agricultural statistics report by state.  Reports are currently available at: https://www.nass.usda.gov/Statistics_by_State/New_Mexico/Publications/Annual_Statistical_Bulletin/ .  Unfortunately, the target printing date for those reports is often later than when needed. If this is an issue, the needed tables can be found sooner at the USDA QuickStats page: https://quickstats.nass.usda.gov/

- Gila National Forest Service office – Marjorie (Marge) Williams, Resource Assistant. Phone 575-388-8282. Email marge.williams@usda.gov or Marjorie.williams@usda.gov. Silver City, NM.  Main office number: (575) 388-8201.

- Annual Deming Water Master Report – The annual report is obtained through the OSE NMWRRS service.  Navigate to the website below, and in the ‘WR File Nbr’ tab, select Gila San Francisco as the basin.  In ‘nbr’, enter ‘0000’.  Click ‘View Water Rights Summary’.  This will open a popup window that will grant access to the Deming Water Master Reports.  Under ‘events’ click on ‘get images’.  A new popup will appear showing the first page of a report.  Click ‘view’, and then click ‘all’.  This may take a while to load, but it will eventually load the entire report.  Save the report to a folder for reference and safe keeping.   http://nmwrrs.ose.state.nm.us/nmwrrs/waterRightSummary.html

- Completed Weather data workbook

- Completed CropStats data workbook.  Ensure the tables are fully QA/QC’d before starting

- Completed Diversion Record workbook

- Completed XirrigCU fortran output tables

The workbook contains 14 sheets.  The first three are for non-irrigation uses like stock, domestic, municipal, and reservoir evap.  The next 9 are irrigation area sheets.  The last two are summary table sheets for reporting.

#### “Tables 1, 2, 3, and 4”

The first place to be updated is in the last sheets in the workbook.  This is fixable if skipped but is easiest if taken care of right away.  Tables 1 and 4 only require updates to the title year on each table.  Table 2 requires an update to the title year and a footnote of consumptive use from Freeport McMoran to Silver City exports.  Make those changes now.

Table 3 is a total of the consumptive use and provides a summary for the last ten years.  It is actively calculating the entire workbook as the ‘current year’ and will become incorrect as soon as updates are made to the other sheets.  To start, copy and paste the values of the previous year on the side (should be line 64, columns B, C, and D).  Delete the 10th year and add a new line for the new year.  Now, the cells being referenced are in column K of table 2, and they sum the total consumptive use for the San Francisco River, Gila exclusive of Virden, and San Simon Creek.  Copy and paste the values for the previous year into the correct place.  Re-reference the cells for the new year to the totals in table 2 above.  The values will be wrong but will become correct when the Rpt20xx sheet is completed.  If this step is missed, find a previous years copy of the Rpt20xx sheet to get the correct values.

#### Stock-dom-com-ind-evap

The purpose of this sheet is to calculate the consumptive use of various sectors - livestock, domestic, industrial, commercial, and evaporation from lakes.

Livestock – Line 1

Lines 4, 5, and 6 ask for the number of cattle and sheep in Grant, Hidalgo, and Catron counties.  Use the USDA Annual Agricultural Statistics report (or QuickStats) to complete this section.  Find the ‘All Cattle’ table in the report.  It should look like the table below.  Do not use the ‘Cattle and calves’ section.  For example, 2020 ‘All Cattle’ for Catron County should be entered as 22,000.

For the Gila National Forest, contact the USFS office to get the number of head for the year.  Forest Service cattle surveys are conducted on a ‘grazing year’ calendar and will be available after March 1 of the following year.  Enter this value in E10.  This completes the input of livestock data, however, there are more updates to be made.  Lines 23, 26, 32, 33, 37, 38, and 48 have annotation cells in column C that explain what is happening in column B.  These values should be updated, and color coded accordingly.

Municipal – Line 53-70

This section requires the Watermaster report.  At the end of each area section in the report (see table of contents), there is a table for the wells titled ‘Non-irrigation Rights – Metered’.  Total the usage to date column and report each value in the corresponding area in the worksheet column C.  This is all the use that is assumed municipal and just non-irrigation.

This is straightforward for all but two areas.  In the Upper Gila, find the value for C63 as per above.  Cell D63 requires the number for the Gila Geothermal well (GSF—02419) to be entered manually into the cell.  Search that well in the watermaster report.  There is a note that details what is required in the cell.

The Cliff-Gila C64 cell requires 4 pieces of information: Freeport total diversion + Silver City diversion from Frank’s Well Field + Diversion for Brushy Mountain + total from the Cliff-Gila area table.  The Freeport total diversion is linked automatically from the Freeport sheet.  The value for Frank’s Well Field can be found in the memorandum in the beginning of the watermaster report.  Section 2, subsection a will have a paragraph that will detail how many acre feet were diverted from Franks. Next, Brushy Mountain AKA the Silver City Radar site should be a 0.  Finally, get the value from the ‘Non-irrigation Rights – Metered’ table and enter it.  This will complete the calculation.

Cliff Gila cell D64 requires the numbers from Frank’s Well Field and the ‘Non-irrigation Rights – Metered’ table that were obtained above.  It also requires the well diversion amount for GSF-00051, Browns Fishpond Diversion.  Enter those values as necessary.

The Virden Valley cell (C70) requires 5 wells to be added together: VV-00106, 139, 145, 152, and 167.  This information may need to be requested from District 3 watermaster directly.

Finally, correct the value on line 72 to include the Geothermal well value.

(With Franks Well Field diversion in mind, go to sheet ‘Tables 1,2,3,4’ and update the annotation cell C48 with that value before moving on.)

Lake Surface Evaporation – Line 76-96

Lake Roberts, Wall, Snow, and Bill Evans Lake evaporation are calculated here.  Using the Diversion record workbook, enter then monthly precipitation amounts from Gila Hot Springs into H79:90.  This will remove the amount of rain that hit the reservoir from total consumptive use.  Predetermined evaporation pan factors will calculate the rest.

Stock Tank Evaporation – Line 97-147

These lines require no updates and pull data in from elsewhere in the workbook.  It applies various evaporation rates and constants to stock tanks in the area.  The resulting data get pulled into other tables from here.

#### “Freeport”

Open the District 3 watermaster’s report and navigate to the end of the Cliff Gila section.  This area contains scanned reporting from Freeport Mcmoran Mine and details their diversions and consumptive use.  The table shown below shows the format for how the data are presented.  The highlighted information is what is needed to complete this section.

#### “Diversions”

This table calculates itself based on other data in the workbook.  It is a final table and requires no updates.

#### Area Sheets

There are 9 area sheets, one corresponding to each irrigated area in the Gila.  This sheet totals the consumptive use by open water evaporation and consumptive use by irrigation.  It also uses the diversion records and weighted CIRs to assess shorted acres.  In the diagram below, there are 4 – 5 areas that need updating, depending on the number of ditches in each area.  The blue box (upper left) is the rainfall, the red boxes (top) are ditches, the yellow (center) is the unmetered use, and the green box (bottom) is the irrigation consumption from XirrigCU.  The brown box (lower right) is a list of groundwater diversions for the Glenwood, Cliff Gila, Redrock, San Simon, and Virden area.

Blue box – For this section, you will need the Weather data workbook.  Enter the monthly precipitation for the irrigation area in D12:23. The values will need to be divided by 12 to convert them to feet. Do this for all areas, 1 – 9.

Red box – Each red box represents a different ditch within the given area, here we see Cliff-Gila as an example.  The CropStats and Diversion workbooks are required to complete these sections.  The data entry required for this section works well if done with the Yellow section as well.  There are three types of data to enter here – the ditch diversions, the reservoir acreage, and the crop acreages.

To start, open the Diversion workbook, and copy/paste the monthly diversions from each ditch into the appropriate column, the leftmost column of each ditch.  For example, the monthly diversions of Gila Farms Ditch would be entered into E12:23.  Preserve the ‘partial data’ formatting from the original source but make sure color code the values with the current year.  Do this for all ditches listed in each area, noting that the number of ditches is not constant across areas.  Reserve area has the most ditches at 7, and San Simon has the least at 0.  Note that Virden Valley has no ditches gaged under the 1964 USSC decree, the Sunset and New Modell are under the Globe Equity.

Next, calculate the reservoir acreage for line 7.  Open the CropStats workbook and navigate to unit-ditch-irr-crop sheet.  Here you will filter the data by ‘Ditch’.  Next, filter by ‘Crop Code’, and select only Res,W AND Stk,W.  This will show only the wet reservoirs and stock tanks for the ditch in question.  Check the remarks column to see how full the feature was at the time of survey.  For example, if a one-acre tank was 80% full at the time of survey, multiply 1 * 0.80 to get the acreage for evaporative loss.  Do this for each feature on the ditch and sum the values together.  Put this number in line 7 under ‘Reservoir Acres’.

Calculate the crop acreage in line 9.  In the CropStats workbook, filter by ‘Ditch’ to the ditch in question, and then filter ‘Irr_code’ to be 1, 2, and 3.  Sum the acreage amounts and enter that number into line 9.

Yellow Box – This box totals all the non-metered irrigation use.  The listed gages at the top do not represent every ditch or diversion in the area, and this is where those uses are estimated.  The difference with this section is that actual shortages are not estimated since there are no data on ditch amounts, these acres are ‘subject to shortage’ simply by being surface water.  Each area has 4 cells that require an update using the CropStats workbook.

The four cells will require different filtering schemes described here:

- M29 – All of the surface water irrigated acres that are not on a gaged ditch.  In CropStats, filter down to a UCode like Luna (LU).  Under ditch, select only ditches that are not present on the Area sheet, and include the blanks.  From here, filter ‘irr_code’ to include only ‘1’ (flood irrigation). The sum is at the bottom of computed acreage.

- P29 – All of the unmetered acres irrigated with groundwater (sprinkler or drip).  Change the above filter setup ‘Irr_code’ to just ‘2’ and ‘3’.  Do not include 1, 4, or 5.

- M31 – All of the reservoirs and stock ponds fed by ungaged ditches.  From the same selection, change ‘crop_code’ to Res/W AND Stk/W.  All irrigation types should be 4, and no changes will separate them.  Examine the remarks, and attempt to see if it is surface water fed.  Any water right with the prefix ‘SD-xxxx’ is a surface right and will apply here.

- P31 – all of the reservoirs and stock ponds fed by groundwater.  From the above selection, use the same reasoning but try to determine if there is an associated well.  LWD-xxxx is a common livestock well permit.  If an absolute answer can not be found, make sure all reservoir acres are accounted for.

For each area, the numbers in S29 (total acres) should match the number in row 54 of the CropStats workbook.  This number will be the sum of all metered and non-metered acres.  Use this as a guide to ensure that all acres are being captured.  If the answer is incorrect, check or reset the filters and try again.

San Simon is entirely groundwater fed, keep this in mind on the yellow box section.

Green Box – These values will come directly from the XirrigCU outputs (See Analyzing the Outputs below).  Line 35 asks for the output of the MBC cropcu.out. This is the last table in each irrigated area (see: WEIGHTED MONTHLY CIRS IN INCHES FOR CROPPING PATTERN), and specifically, it is asking for the monthly values of WCIR to be copied into line 35.  Line 41 asks for the output of the total WCIR in the OBC table in its cropcu.out.  This one value will be distributed across the months in the same ratio that the MBC was.  Do not copy over the monthly values from OBC.

Brown Box – This box will be in the lower right of the Cliff Gila, Redrocks, Virden, and San Simon sheets.  To fill this out, use the district 3 watermaster’s report to find the information.  These are wells that are specifically irrigation wells within each area.  If they aren’t found in the watermaster’s report, reach out to District 3 to get the information.

## Reporting letter

There are two documents required for updating, the Transmittal Letter and the Annual Report.  The language on the Transmittal Letter should be taken from a previous version and updated to the current year.  This should be copied and pasted to a current version of ISC letterhead, with the current commission members.  The Annual Report can be found with a previous year as well.  There are no specific data in this report.  Perform a find-replace to update all the year statements and save it.

Take the tables 1,2,3,4 sheet from the Rept20xx workbook, and save it into a word document.  Format as necessary.  The following files should be ready for their respective parties.  Submit the material to internal NMISC before dissemination.

For Reclamation

Transmittal Letter

Annual Report

Table I – IV Report

For OSE Website

Gila Area Weather workbook

Ditch Diversion Workbook

Crop Statistics Workbook

Gila Report Data

Compile the documents together, and mail a copy to Reclamations Lower Basin Office at:

Email copies to Jeremy Dodds at:

Create Excel workbooks suitable for posting to the OSE website

http://www.ose.state.nm.us/Basins/Colorado/Gila/isc_Gila.php

Remove any personal names.  Remove “Remarks” column from “20xx Gila Crop Statistics”.

Password-protect each worksheet using “r5t6Y&U*”.  Allow users only to “Select locked cells” and “Select unlocked cells”.  In the “20xx Gila Report Data” workbook, on the “Table1,2,3,4” worksheet, also allow “Format columns” so that users can get to the hidden columns.

IT uses the RealFile site, which is the backend of the main OSE page.  As of 2023, the link below is current, but will require user/password access from IT.  When opening, use the following filepath: ISC Basins -> Colorado River (San Juan Basin/ Gila) -> Gila River Basin.  There will be four folders: one for Crop statistics, Ditch Diversions, Gila Report Data, and Weather.  Add the protected spreadsheets to the designated folders.

https://prod.realfile.rtsclients.com/login.html?domain=ose.rtsclients.com&clientGUID=381810c4-dc4c-447d-af8c-d5901ad11b91&serviceURL=https%3A%2F%2Frf-sb-prod.rtsclients.com%2F

# Miscellaneous

# Troubleshooting XirrigCU

There are three main types of ‘errors’ produced with XirrigCU – no outputs, blank outputs, and incorrect data.  Very few, if any, formal errors will be presented to the user.  The following will detail some known types of errors, and their likely causes.  This list should be added to as more are found.

- Use spaces, never tabs.  Copying and pasting values from Excel is a good practice, however all spaces between values should be deleted and manually re-spaced.  Excel will copy cells as tab separated.  Also, do not leave blank lines between sections.  Inadvertent tabs may result in blank outputs. Inadvertent lines may result in blank outputs.

- Notepad is the preferred program to ensure everything is in order.  Notepad++ has helpful features and can be used for data entry, but a final examination before running XirrigCU should be done in Notepad to ensure correct formatting.  Notepad++ can insert tabs instead of spaces.  The representation seen in Notepad will better resemble how XirrigCU will see the information.

- When running XirrigCU, copy the filename of the control entirely and exactly.  Forgetting the .DAT will result in an error.  Using an incorrect filename (i.e., incorrect year) will result in an error and no outputs.

- Make sure the ‘rows’ and ‘columns’ are matched to previous years closely.  In the CDF, the location of the decimal relative to the data is key for the crop input.  On irrigation pattern, it is seen to change throughout some years.  Wherever possible, fit the format of the previous year.  Also be sure to match the number of significant figures as seen in previous years.  If this is done incorrectly, the program may run successfully but render incorrect information.  Use the following check to ensure the run went well.

- The cropsum files are a critical post processing check.  On each run, ensure the Crop Distribution Ratio (CDR) for each crop types is 1.000.  If it is different than that, then the program is reading an incorrect number of acres than it is reporting, and the CU will be wrong.  Note that the ‘acreage’ outputs and the CDR will be interpreting two different numbers.

- Make certain that the crop and irrigation pattern in the CDF equal 100%.  If there are more than 100% represented, you will not receive an error, just bad information.

- Sometimes nothing can be done to fix the issue.  In that instance, it may be valuable to simply start over with an archived copy of the inputs and re-enter data to see if that fixes it.

## Irrigated acreage locale and their associated weather stations

#### Gila Weather Station

| Crop Data File (CDF) Area | Name | WX area | Name |
| --- | --- | --- | --- |
| 1 (GSLU) | Luna | 1 | Luna (Uses Luna Ranger Station) |
| 2 (GSAR) | Apache-Aragon | 2 | Apache-Aragon (Beaverhead, Reserve, Luna Ranger Station) |
| 3 (GSRE) | Reserve | 3 | Reserve (Uses Reserve/Hood Ranger Station) |
| 4 (GSGW) | Glenwood | 4 | Glenwood-Mule Creek (Uses Glenwood Ranger Station) |
| 5 (GSUG) | Upper Gila | 5 | Upper Gila (Uses Gila Hot Springs Station) |
| 6 (GSCG) | Cliff-Gila | 6 | Cliff-Gila-Buckhorn-Duck Creek (Cliff 11 Se or Buckhorn Station) |
| 7 (GSRR) | Redrock | 7 | Redrock (Uses Redrock Station) |
| 8 (GSVV) | Virden Valley | 8 | Virden Valley (Uses Station at Duncan Az) |
| 9 (GSSS) | San Simon | 9 | San Simon (Uses Animas Station) |